Technisonic TDFM-136 Radio

Power Up

- To power up the Technisonic TDFM radio make sure the Mission Master toggle switch is in the on position.  This switch is located just to the right of the co-pilots side control yoke.

- Desired Settings:

  - Main switch is turned on and set to a mid-volume level.

  - Guard volume switch is turned down.

  - MN/GD switch is set to MN.

  - HI/LO switch is in the LO position (use HI power only if needed, HI can distort or “motor-boat”)

- If you get in trouble with a command, press the */ESC key to escape the command.

- Use COM3 to listen and transmit on FM.

- Press TDFM Squelch to verify you are listening to COM3 and to set the “Main” Volume level.

Display Brightness

- To adjust the display brightness, use the 2 and 8 keys to increase/decrease the display brightness

Channel Selection

- The FM radio is pre-programmed with appropriate CAP frequencies.  See the communications section of the aircraft book for the channel plan.

- To scroll through channels use the 4 and 6 keys to scroll backwards and forwards through the channels.

- To select a channel by channel number, press the 1/CHAN key and enter a 3 digit channel (001 to 200) using the numeric key pad.  To accept the entry, press the #/ENTER key.  To abandon the selection, press the */ESC key.  Note the MN/GD switch MUST be in the MN position.

- Do not use the 7/FREQ key unless you know what you are doing.  If in doubt, use the */ESC key to get out of any command.

- If the keypad appears not to be responding, it may be locked.  To unlock the keypad, hold the */ESC key for two seconds.   This should unlock the keypad.

Talking on the radio

|  | - The TDFM radio is COM3 on the audio panel. - To monitor the FM radio, press “COM3” to illuminate it. Remember to do this on both audio panels for everyone to be listening to COM3 - To transmit on the FM radio, press “COM3 MIC”. Note the selected MIC sets that COM to receive mode. - The PTT switch will now transmit on the FM radio when pressed. |
| --- | --- |