Basic G1000 Mission Observer Quick Reference Guide

KEY:

B		Turn big (outer) knob

S    		Turn small (inner) knob

RANGE	Press named knob or softkey

ENT	   	Press named hardkey

All Tasks start at Map Page 1 (hold CLR for 2 seconds)

Mark and Save a Point

- Press RANGE once.

- Within 5 seconds press ENT

- B to “Create User Waypoint”

- ENT

- S left to ‘1’ (for first, ‘2’ for second…)

- ENT

- Hold CLR for 2 seconds  Map Page 1

Retrieve & Delete Waypoint (post-flight)

- B to WPT page group.

- S to “User Waypoint” page.

- Press S knob to get cursor.

- B down to “User Waypoint List”, continue to “1”.

- Write down “Information” (Lat/Long info)

- DELETE softkey

- ENT to verify delete

- <repeat for all numbered waypoints>

Show Quarter Grid Lat/Longs

- Hold CLR for 2 seconds  Map Page 1

- Turn RANGE knob until Map Scale shows “30nm”.

- If  lat/long lines are already showing, skip next Task “Enable Map Lat/Longs”.

Enable Map Lat/Longs

- MENU

- B to Map Setup

- ENT

- S to “Land”

- ENT

- B to “LAT/LON”

- Using B to move field to field, S to modify a field, and ENT to save:  Set ‘TEXT’ = Small, ‘RNG’ = 30nm

Mission Master – power on CAP-specific equipment (FM, DF)

FM Radio Setup

Main = Volume – set about halfway as shown Guard – turn all the way down (counter-clockwise) Squelch – press to verify you can hear the radio (COM3) White switches – up up down as shown Keypad: up / down arrows (2, 8) to set brightness left / right arrows (4, 6) to go up / down channels

Basic G1000 Mission Observer Quick Reference Guide

| Select Switches Setup |  |
| --- | --- |
| Good – both Green | Bad – either one blue |

| Comm Audio Panel Setup |  |
| --- | --- |
|  | - Mic – select for transmitting - Right side COM1, COM2, COM3 – all should be selected for listening - Also select AUX to listen to Becker DF |

Becker Direction-Finding Setup

- (1) Power on/off (or use Mission Master) - During power-up, use Page Knob (2) to set Emergency or Training mode. You only have three seconds.

- Use bottom right knob (1) to select Frequency - Use Volume (2) to set Volume - Use SQL (3) to set Squelch level (triangle in the black) - Listen by selecting AUX on Comm audio panel - If no signal is heard, try listening on COM2

Advanced G1000 MO Quick Reference Guide

Grid Search – Mission “Fast Launch” – set one corner

KEY:

B		Turn big (outer) knob

S    		Turn small (inner) knob

RANGE	Press named knob or softkey

ENT	   	Press named hardkey

All Tasks start at Map Page 1 (hold CLR for 2 seconds)

See next page for how-to modify all 4 grid points & create flight plan

Do this after Engine Start, before takeoff.  Eyes outside while plane is taxiing – do this only while plane is stationary

Clear all Flight Plans

- FPL

- S to “Flight Plan Catalog” page.

- MENU

- S  to “Delete All”

- ENT

- ENT

Modify 1 existing Waypoint:  GPTNW

- Go to Map Page 1 (hold CLR for 2 seconds)

- B to WPT page group.

- S to “User Waypoint” page.

- Press S knob to get cursor.

- B down to “User Waypoint List”, continue to GPTNW.

- ENT to get to detailed info & modify the waypoint

- B to “Information” area.(lat/long values)

- S right once to start editing.

- Using B to move field to field, and S to modify a field, edit the lat/long.

- When correct:  ENT to save.

Advanced G1000 MO Quick Reference Guide

Grid Search – Sarex “Slow Launch”

Modify all 4 Waypoints, create flight plan “grid box”

Do this after Engine Start, before or after takeoff.  Eyes outside while plane is taxiing – do this only while the airplane is stationary

KEY:

B		Turn big (outer) knob

S    		Turn small (inner) knob

RANGE	Press named knob or softkey

ENT	   	Press named hardkey

All Tasks start at Map Page 1 (hold CLR for 2 seconds)

Clear all existing Flight Plans

- FPL

- S to “Flight Plan Catalog” page.

- MENU

- S  to “Delete All”

- ENT

- ENT

Modify 4 existing Waypoints (eg GPTNE)

- B to WPT page group.

- S to “User Waypoint” page.

- Press S knob to get cursor.

- B down to “User Waypoint List”, continue to “GPTNE”.

    - ENT to get to detailed info & modify.

    - B to lat/long area.

    - S right one to start editing.

    - Using B to move field to field, and S to modify a field, edit the lat/long.

    - When correct:  ENT to save.

    - B right to next waypoint, eg GPTNW.

    - <Repeat for all 4>

- Hold CLR for 2 seconds  Map Page 1

- RANGE out, PAN*, verify 4 GPT waypoints form a square.

Create a new Flight Plan

  - FPL

  - Press S knob to get cursor.

    - B to put cursor on blank line.

    - S right once to add a waypoint.

    - S left once to get to shortcuts

    - S right to “RECENT”

    - B right (down) to desired waypoint name

    - ENT

    - ENT

    - <Repeat for all waypoints>

Store a Flight Plan

- FPL

- S to “Active Flight Plan” page.

- MENU

- B to “Store Flight Plan”.

- ENT  (dialog asks to store to #1?)

- ENT   (yes, store it)

*PAN*

Press RANGE to get arrow pointer. Move “joy stick” to move pointer on map