T182T Nav III Transition (Add-On) Form 5  Expectations

The G1000 add-on Form 5 shall follow the evaluation items in the FITS training syllabus and the Form 5.  Common G1000 tasks include:

- Entering flight plan and storing (need avionics 2 on and ENT’d)

- Adding an Along-Track-Offset waypoint

- Use of autopilot for climb, level-off

- VNAV setup

- Entering com and nav frequencies using G1000 database

- Use of autopilot for VNAV descent

- Able to bring up XM weather, eg METAR/TAF, forecast winds aloft, cloud cover, TFR’s

- Set MFD map to north up, desired track up, or track up

- MFD turn on / off terrain, topo

- MFD traffic alert – given alert (circle & line), which direction is the target going?

- Inset map setup – set range (5-8 nm), turn on traffic, terrain, topography, stormscope.

- Use inset map changing yellow -> brown to signify 1000 AGL => time to engage autopilot

- Know where to find:

  - Current winds aloft at our altitude

  - Current outside temperature

  - Current TAS

  - Current ground speed

- Use of checklists for climb, cruise, descent

- Use of Engine -> Assist for leaning at cruise

- Diversion – use of NRST on PFD, enter Unicom frequency from NRST

- Unusual attitudes / hood work per Private PTS

- Loss of PFD or AHRS/ADC.  Be able to land on standby instruments

- Name at least 3 ways to disconnect the autopilot

T182T Nav III Annual Form 5  Expectations

Enter Flight plan and store during preflight

Standby battery Test – 20 seconds (count fast)

Runup

- Flight plan activate

- Add VNAV offset:  minus 3 miles, 2° FPA maximum

- MET memorized

- Setup PFD Inset map – 5nm range or more

- Setup PFD right inset to Fplan or as desired

Climbout – engage autopilot

1. Set heading bug on runway heading before starting takeoff roll.

2. Climb straight out; do not make cross-wind turn.

3. Engage autopilot, runway heading, 1000 AGL, 85+ KIAS.

4. After establishing autopilot climb above pattern, turn to intercept course or desired track using the heading bug.

Autopilot engage – no delay on getting to up/down.  Select Altitude Arm and Nav before reaching cruising altitude and course.

Climb checklist – use – this turns on Mission Master

Cruise checklist – use

- Lean must use Assist not TIT over G.

- Ok to cruise 20-22” for more time to think

Airwork, unusual attitudes, basic instrument flying, takeoffs and landings, simulated forced landing – to PTS standards

Before Top of Descent

- Recheck VNAV for FPM / FPA

- Put AWOS, Unicom in active using G1000

- Have AWOS and announce intentions 5-10 miles out

Top Of Descent

- Plan ahead

- Set autopilot for new level-off altitude (TPA)

- 400 fpm (set on autopilot) = reduce ≥4” MP

- Descent checklist

- Radio call

Fail AHRS or turn down PFD brightness

- EP checklist right pocket

- NRST -> put in frequency

- Can’t put in AWOS without MFD

- D-> Nearest

- Can you use the AP?

IP/CP - If TIT ever blinks red fix now, do not wait for pilot-in-training to notice

Go-around is always ok – shows good judgment