T182T GPU – based training

1.  GPU…in box south side of hangar.

A.  14/28 volt…be sure to use 28v.

B.  On/Off switch. “On” after plug in to A/C. “Off” before pulling plug.

2.  Aircraft.

A.  External power door pilot side of cowling.

1.  Phillips head driver to open

2.  Power plug is keyed…”can’t” plug in wrong.

B.  A/C master, STBY pwr sw,  and GPU pwr switch OFF when connecting or 	      	      disconnecting.

1.  GPU replaces Main Battery…disconnects it from A/C system. System 			     load will be read as negative amps on the EIS MB ammeter.

2.  Refer to POH electric schematic?

3.  Procedures.

A.  Normal preflight (no GPU) up to engine start.

B.  Simulate engine start…plug in GPU and turn on.

1.  Make sure A/C master & STBY pwr sw are OFF for this.

C.* After GPU on MB (master) sw ON and avionics 2 & 1 sws ON.

1.  Probably doesn’t matter if ALT sw is on or off.

2.  Suggest avionics 2 and then 1 for fan recognition.

3.  If STBY pwr sw now armed, the GPU will charge the stby batt.

D.  PFD/MFD familiarization…flow chart from G1000 manual?

1.  Initial screen info on power up.

2.  Screen options…PFD/MFD.

3.  Flight plans.

E.  Shutdown.

1.  Shut down A/C per check list.

2.  Turn OFF and disconnect GPU.

3.  Secure external power door.

*  Start procedures here if you don’t want to go through the preflight.

Autopilot??  I’m sure that engaging it on the ground for any length of time is not good 	for it so it seems any serious training must be done in flight. Any ideas??