T-182T Nav III  Procedures

T/O on runway,

Sync Heading on G1000

Mixture rich minus 3  ½ turns lean

Throttle to 25” pause till spool up

Throttle to 32 “ ( almost full  ~33)

After obstacle cleared :   Flaps up

Auto pilot Engage Pre-Requisites

- Trimmed airspeed >85 kts

- 1000+ AGL-

- Sync HDG on G1000

- Flaps up

- Throttle 25”, mixture  16 Gal/hr

Autopilot Engage

AP on – 2 sec

Press Hdg on AP

Press Up/down to set 300-400 fpm

Press Arm

(Left to Right Flow: AP, Hdg, Up/Down, Arm)

Slow flight- MCA:  12” or less  to slow down then:

Flaps		    Power setting

0			22x17

- 22x18

- 22x18

Full		22x24

Stall recovery (climb, clean, cool)

(prop should be forward)

Throttle to 25 “

[flaps to 20 if full][> 70 kts retract remainder]

{ If more power needed or desired

Mixture to rich minus  3  ½ turns lean

Prop full forward (if not already)

Throttle to 32”}

Cowl flaps open

Steep turns

22-23 x 25”

Power setting 	Speed

22x20			105

22x18			100

22x17			 95

22x16			 90

+10 Flaps

22x18			90

22x17			85

22x16			80

Go around ( climb, clean, cool)

(prop should be forward)

Throttle to 25 “

[flaps to 20 if full][> 70 kts retract remainder]

{ If more power needed or desired

Mixture to rich minus  3  ½ turns lean

Prop full forward ( if not already should have been!)

Throttle to 32”}

Cowl flaps open

Stop and go’s  better (or touch and go’s)

Reconfigure (flaps and trim)

Open cowl flaps

Mixture to rich minus  3  ½ turns lean

Verify prop full forward (if not do it now)

Throttle 32” or pause at 25”

Quick reference mixture setting

Set TIT arrow right over G in Gal gives ~ 1585

Rev 1.0  14 Mar 2008