G1000 Mission Observer Task-Based Quick Reference Guide

Complete as many tasks as possible in this box within 15 minutes to conserve the aircraft battery

KEY:

B	Turn big (outer) knob

S    Turn small (inner) knob

RANGE  Press named knob or 	softkey

ENT	   Press named hardkey

All Tasks start from Map Page 1

When in doubt, CLR it out

Hold CLR for 2 seconds  Map Page 1

Power on aircraft

Master ALT and BAT on.

Avionics on:  BUS 2 then BUS 1, listen for each fan.

PFD and MFD show “Initializing System”, NO

Check database dates, ENT

Clear all existing Flight Plans

FPL

S to “Flight Plan Catalog” page.

MENU

S  to “Delete All”

ENT

ENT

Modify existing Waypoint (eg GRSE)

B to WPT page group.

S to “User Waypoint” page.

Press S knob to get cursor.

B down to “User Waypoint List”, continue to GRNE.

ENT to get to detailed info & modify.

B to lat/long area.

S right one to start editing.

Using B to move field to field, and S to modify a field, edit the lat/long.

When correct:  ENT to save.

B right to GRNW.

<Repeat for all 4>

Hold CLR for 2 seconds  Map Page 1

RANGE out, PAN*, verify 4 GR waypoints form a square.

Create a new Flight Plan

FPL

Press S knob to get cursor.

B to put cursor on blank line.

S right one to add a waypt.

S right one to start editing.

Using B to move field to field, and S to modify a field, enter GRNE etc.

ENT

<Repeat for all waypoints>

Store a Flight Plan

FPL

S to “Active Flight Plan” page.

MENU

B to “Store Flight Plan”.

ENT  (dialog asks to store to #1?)

ENT   (yes, store it)

Power off aircraft

Avionics BUS 1 & 2 off

Master ALT and BAT off

*PAN*

Press RANGE to get pointer. Move “joy stick” to move pointer on map

Retrieve, Activate a Stored Flight Plan

FPL

S to “Flight Plan Catalog”

Press S knob to get cursor

B to desired flight plan

ACTIVE softkey to Activate

ENT

Show Quarter Grid Lat/Longs

Hold CLR for 2 seconds  Map Page 1

Turn RANGE knob until Map Scale shows “30nm”.

If no lat/long lines showing, go to next Task “Enable”.

Enable Map Lat/Longs

Map should be on 30nm range to see quarter grids.

MENU

B to Map Setup

ENT

S to “Land”

ENT

B to “LAT/LON”

Using B to move field to field, S to modify a field, and ENT to save:  Set ‘TEXT’ = Small, ‘RNG’ = 30nm

Mark and Save a Point

Press RANGE once.  Within 5 seconds:

ENT

B to “Create User Waypoint”

ENT

S left to ‘1’ (for first, ‘2’ for second…)

ENT

Hold CLR for 2 seconds  Map Page 1

Retrieve & Delete Waypoint

B to WPT page group.

S to “User Waypoint” page.

Press S knob to get cursor.

B down to “User Waypoint List”, continue to “1”.

Write down Lat/Long info

DELETE softkey

ENT to verify delete

<repeat for all numbered waypoints>